| SC4QC-1 |  | 2 |  | Ed | References in Clause 2, that should be moved or added to the bibliography because they are NOT cited in the text in such a way that some or all of their content constitutes requirements of the document. ISO 10303-1 should be moved to the Bibliography ISO 10303-11 should be moved to the Bibliography ISO 10303-1509 should be moved to the Bibliography ISO 10303-1514 should be moved to the Bibliography ISO 14649-1 should be moved to the Bibliography ISO 286-1 should be moved to the Bibliography ISO/IEC 8824-1 should be moved to the Bibliography | Move to bibliography | RESOLVED (ED) - Adopted proposed solution where appropriate. ISO 10303-1, ISO 10303-11, ISO 14649-1 not moved are in normative text in clause 3. ISO 286-1 not moved, is in normative text for Drill_and_fill_prototype.iso_fit. ISO/IEC 8824-1 not moved because it is in normative Annex E. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SC4QC-2 |  | 2 |  | Ed | References in Clause 2, that cannot be found in the documentation, please check. ISO 10303-1131 ISO 10303-1796 ISO 10303-1797 ISO 10303-44 ISO 1101 ISO 23247-1 ISO 2806 ISO 286-2 ISO 5459 | Remove | RESOLVED (ED) - Adopted proposed solution where appropriate. ISO 1101, ISO 23247-1, ISO 2806, ISO 5459 are referenced in clause 3 |
| SC4QC-3 |  | 4, 5 |  | Ed | Do not use these provisions in the fragments displayed below. Use only the provisions listed in IDP2, Clause 7. clause4 NOTEThe constant power density is controlled on the machine so the value of (C) does not need to be programmed. clause4 The backside of a robot may need to point away from a fixture. clause4 An External_operation application object is a type of Operation that is performed by an external system that is beyond the scope of the automation defined in this document but whose completion needs to be recorded in the digital twin. annexK A planning_operation has been added to Program_structure to describe information that could be used to generate workingsteps just before or during manufacturing. clause4 In addition, only length parameters could be qualified with a tolerance. clause4 EXAMPLEA Feature_complete_probing could adjust the position of a product such as a fastener, or a process such as a drilling operation. clause4 The message could be put into a PPRINT command to show that the broadcast has been processed. clause4 The measured properties could be used for values that are described by an external standard. clause4 The current operator id could be held in the location such as "OPERATOR_BUFFER[68]". clause4 A drill and fill executable could add a fastener, collar or washer. clause4 Also, when drilling on a machine with redundant (not necessarily parallel) linear axes, such as a machine with a movable quill (W axis), either the W or other axes could be used to advance the drill. clause4 A material workpiece could also be a Product_view_twin if it includes measured properties specific to an instance of the material. clause4 NOTE The planning operation describes information that could be used to generate workingsteps just before or during manufacturing. clause4 This will be of benefit to related downstream tasks, such as inspection, which could use the tracability to pinpoint critical areas of interest in the micro and macro process-plans. clause5 Before it is machined, the top of a feature often corresponds to a tangible location on the stock where a machinist could measure or make a mark. clause5 This associativity through the entire design to manufacturing path is useful for related downstream tasks, such as inspection, which could use the tracability to pinpoint critical areas of interest in the micro and macro process-plans. clause5 A feedrate could be described either as a linear speed (length per unit of time), or as a length per tooth of the tool being used. clause5 The model could represent the values, but could not distinguish between as-planned and as-executed data sources. clause5 Furthermore, the model could only represent as-executed values for a single execution run. scope Future editions of this document might include additional processes if additional ISO 14649 descriptions become available. clause4 The its_technology may not be specified for a particular Am_operation. clause4 If compute_downskin is false or unspecified, they may not. clause4 If compute_upskin is false or unspecified, they may not. clause4 If compute_spike is false or unspecified, they may not. clause4 If compute_thin_wall is false or unspecified, they may not. clause4 The its_support_geometry may not be specified for a particular Executable. clause4 PLC measurements may not be portable between machines. clause4 The clamping shall be performed at high speed if high_speed is true, and may not if high_speed is false or not specified. clause4 The its_toolref_direction may not be specified for a particular Cutter_contact_trajectory. clause4 The path_maximum_deviation may not be specified for a particular Cutter_contact_trajectory. clause4 The tool_axis_maximum_deviation may not be specified for a particular Cutter_contact_trajectory. clause4 The its_toolref_direction may not be specified for a particular Cutter_location_trajectory. clause4 The path_maximum_deviation may not be specified for a particular Cutter_location_trajectory. clause4 The tool_axis_maximum_deviation may not be specified for a particular Cutter_location_trajectory. clause4 The as_is may not be specified for a particular Executable. clause4 The enabled may not be specified for a particular Executable. clause4 The machine_used may not be specified for a particular Executable. clause4 The process_properties describe characteristics of an Executable that may not be conveyed using other capabilities of this document. clause4 The removal may not be specified for a particular Executable. clause4 The to_be may not be specified for a particular Executable. clause4 The twin_end may not be specified for a particular Executable. clause4 The twin_exception may not be specified for a particular Executable. clause4 The twin_plan may not be specified for a particular Executable. clause4 The twin_source may not be specified for a particular Executable. clause4 The twin_start may not be specified for a particular Executable. clause4 The twin_worktime may not be specified for a particular Executable. clause4 The its_machine_functions may not be specified for a particular Extended_operation. clause4 The its_technology may not be specified for a particular Extended_operation. clause4 The its_tool may not be specified for a particular Extended_operation. clause4 The expected_value may not be specified for a particular Feature_complete_probing. clause4 The description may not be specified for a particular Fixture_usage. clause4 The its_product may not be specified for a particular Fixture_usage. clause4 The mount_reference may not be specified for a particular Fixture_usage. clause4 The workpiece_reference may not be specified for a particular Fixture_usage. clause4 The axis_travel may not be specified for a particular Machine_parameters and the axis_travel shall not be specified if work_volume_length, work_volume_width, or work_volume_height is specified. clause4 The feedrate may not be specified for a particular Machine_parameters. clause4 The number_of_control_axis may not be specified for a particular Machine_parameters. clause4 The number_of_simultaneous_axis may not be specified for a particular Machine_parameters. clause4 The positioning_accuracy may not be specified for a particular Machine_parameters. clause4 The spindle_power may not be specified for a particular Machine_parameters. clause4 The spindle_torque may not be specified for a particular Machine_parameters. clause4 The spindle_speed may not be specified for a particular Machine_parameters. clause4 The table_indexing may not be specified for a particular Machine_parameters. clause4 The table_length may not be specified for a particular Machine_parameters. clause4 The table_width may not be specified for a particular Machine_parameters. clause4 The work_volume_height may not be specified for a particular Machine_parameters and the work_volume_height shall not be specified if axis_travel is specified. clause4 The work_volume_length may not be specified for a particular Machine_parameters and the work_volume_length shall not be specified if axis_travel is specified. clause4 The work_volume_width may not be specified for a particular Machine_parameters and the work_volume_width shall not be specified if axis_travel is specified. clause4 The description may not be specified for a particular Machine_usage. clause4 The its_product may not be specified for a particular Machine_usage. clause4 The configuration may not be specified for a particular Machine_with_kinematics. clause4 The its_usage may not be specified for a particular Machining_tool. clause4 If primary is false or not specified, a referencing Drill_and_fill_group may not be preferred. clause4 The property_date may not be specified for a particular Process_property. clause4 If seal_check is false or unspecified, the sealant may not be checked. clause4 The description may not be specified for a particular Tool_usage. clause4 The gage_placement may not be specified for a particular Tool_usage. clause4 The its_carousel may not be specified for a particular Tool_usage. clause4 The its_library_reference may not be specified for a particular Tool_usage. clause4 The its_position may not be specified for a particular Tool_usage. clause4 The its_product may not be specified for a particular Tool_usage. clause4 The tool_end_placement may not be specified for a particular Tool_usage. clause4 The its_id may not be specified for a particular Toolpath. clause4 The ball_radius may not be specified for a particular Touch_probe. clause4 The stylus_diameter may not be specified for a particular Touch_probe. clause4 The its_material_removal_depth may not be specified for a particular Trajectory. clause4 The its_material_removal_overcut may not be specified for a particular Trajectory. clause4 The its_material_removal_total_volume may not be specified for a particular Trajectory. clause4 The cross_section_area_parameters may not be specified for a particular Trajectory. clause4 The toolpath_orientation may not be specified for a particular Workingstep. clause4 The its_approvals may not be specified for a particular Workpiece. clause4 The its_categories may not be specified for a particular Workpiece. clause4 The its_datestamps may not be specified for a particular Workpiece. clause4 The its_orgs may not be specified for a particular Workpiece. clause4 The its_people may not be specified for a particular Workpiece. clause4 The its_timestamps may not be specified for a particular Workpiece. clause4 The product_approvals may not be specified for a particular Workpiece. clause4 The product_datestamps may not be specified for a particular Workpiece. clause4 The product_orgs may not be specified for a particular Workpiece. clause4 The product_people may not be specified for a particular Workpiece. clause4 The product_timestamps may not be specified for a particular Workpiece. clause4 The revision_approvals may not be specified for a particular Workpiece. clause4 The revision_datestamps may not be specified for a particular Workpiece. clause4 The revision_id may not be specified for a particular Workpiece. clause4 The revision_orgs may not be specified for a particular Workpiece. clause4 The revision_people may not be specified for a particular Workpiece. clause4 The revision_timestamps may not be specified for a particular Workpiece. clause4 The its_minimum_machine_params may not be specified for a particular Workplan. clause4 The toolpath_orientation may not be specified for a particular Workplan. clause5 However, the features on the final part may not exactly match the features as machined. clause5 An unplanned operation may not originate in a CAM system. clause5 A machining_final_feature_relationship is a type of action_method_relationship that represents a feature visible on the final form of the workpiece that a workingstep may not manufacture directly, but in some way contributes to the creation of. clause5 The unit for an area measure is described as a derived_unit, but a complex instance of area_unit and derived_unit is not permitted by the EXPRESS definition. clause5 The unit for a volume measure is described as a derived_unit, but a complex instance of volume_unit and derived_unit is not permitted by the EXPRESS definition. |  | RESOLVED (ED) - Adopted IDP2 clause 7 wording Could -> can May not be specified -> may be omitted |
| SC4QC-4 |  | 4, 5 |  | Ed | Do not use "must" as an alternative for "shall"". See IDP2, Clause 7. annexJ The processing system must generate toolpaths from the supplied features in order to machine the part. annexJ The processing system must generate toolpaths from the supplied features in order to machine the part. clause4 In Figure 19, X1 may escape to X2 on level 1 using feature group 1, but must go to level 2 to reach X5 using feature group 4, and level 3 to reach X7 using feature group 6. clause4 If conditions must be applied sequentially, the Representation_items shall be nested, in the required order, within a List_representation_item. clause4 - must_follow. clause4 SUBTYPE OF (Drill_and_fill_group); must_follow: SET clause4 4.3.229.1 oneup_assembly_group to executable (as must_follow) clause4 The must_follow specifies a collection of Executable objects. clause4 At least one Executable must finish before work on this group begins. clause4 At times, there may be many Executables at least one of which must finish before work begins. clause4 In the most complex case an Executable in the set may be a Workplan that describes multiple things that must be completed before beginning the oneup assembly group. clause4 NOTE In the precedence group all must complete, and in the same_assembly group only one must complete. clause4 In some applications a guide is considered stale after 12 hours and must be computed again. clause5 Finally, there are information requirements which were not considered by ISO 14649 for the isolated case of input to a CNC control, but which must be considered by this document for integrating across the lifecycle such as geometric dimensions and tolerances. clause5 One must then geometrically evaluate the distance between this placement and the depth surface to determine the depth of material. clause5 To determine the depth of the hole, one must geometrically evaluate the distance in the negative Z direction between this placement and the depth surface. clause5 To determine the depth of the pocket, one must geometrically evaluate the distance in the negative Z direction between this placement and the depth surface. clause5 To determine the depth of the Profile_feature, one must geometrically evaluate the distance in the negative Z direction between this placement and the depth surface. clause5 To determine the depth of the slot, one must geometrically evaluate the distance in the negative Z direction between this placement and the depth surface. clause5 To determine the depth of the step, one must geometrically evaluate the distance in the negative Z direction between this placement and the depth surface. clause5 The items set must refer to representation_item instances, so measurements are encoded as a measure_representation_item, which is a subtype of measure_with_unit and representation_item. clause5 Local rules on the ISO 10303 geometry definitions require that any enclosing representation must refer to a geometric_representation_context to identify the dimensionality (2D, 3D, etc.) of the geometry. clause5 The definitions of Cutter_contact_trajectory, Cutter_location_trajectory, Axis_trajectory, and Toolpath_speed, require that the curves that describe location (basiccurve), tool axis, surface normal, and feed speed must have the same parameterization. clause5 Where a planned process would produce results, such as touch_probing, the operations identify an external variable name, but the value must be collected from a machine tool by some unspecified method. clause5 Extra via points between two non via points must define arcs of the same radius. clause5 -- the types on either side of the initial value relationship must match -- each nc_variable has at most one one initial value |  | RESOLVED (ED) - Adopted IDP2 clause 7 wording |
| SC4QC-5 |  | 4, 5 |  | Ed | Clause incorrectly used Use Clause only when referring to a complete Clause In clause4, please verify 'The mapping specification given in clause 5.1 defines how the integrated resources and application interpreted constructs are used to meet the information requirements of this application protocol.' In clause5, please verify 'NOTE It is customary to present mapping specifications in alphabetical order by UoF name, however this document presents the mapping specification in the same order as they appear in clause 4.2.' |  | RESOLVED (ED) - Removed "clause" |
| SC4QC-6 |  | 4,6,K,G |  | Ed | Clause, Annex, Figure, Table, and Formula should start with a capital letter when used as references. In annexG, please verify 'The application reference model is a graphical representation of the structure and constraints of the application objects specified in clause 4, including the application objects referenced from the ISO 10303-1000-series parts, ISO 14649-10, ISO 14649-11, ISO 14649-12, ISO 14649-17, ISO 14649-111 and ISO 14649-121.' In annexK, please verify 'Change the measure UoF to reference application object definitions and mappings from ISO 10303-1054, 10303-1106, 10303-1753, and 10303-1782 rather than define them in clause 4 and 5.' In annexK, please verify 'Move the Manufacturing_feature application object from the workpiece UoF into the manufacturing feature UoF and reference the application object definitions and mappings from ISO 10303-1814 rather than define it in clause 4 and 5.' In annexK, please verify 'Change the manufacturing feature UoF to reference the following feature application object definitions and mappings from ISO 10303-1814 rather than define them in clause 4 and 5.' In annexK, please verify 'Also remove definition in clause 5 for chamfer_requires_faces_or_features, edge_round_requires_faces_or_features, restrict_unneeded_feature_usage.' In annexK, please verify 'Delete the turning feature UoF, moving the following application objects to the manufacturing feature UoF. Reference the definitions and mappings from ISO 10303-1814 rather than define them in clause 4 and 5.' In annexK, please verify 'Added machining_twin_relationship to clause 5 for use in the mapping and guidance on Digital twin process in the Fundamental concepts and assumptions section.' In annexK, please verify 'Added machining_fixture_usage to clause 5 for use in the mapping.' In annexK, please verify 'Add extended_type_operation to clause 5 to support mapping of Extended_operation.' In annexK, please verify 'Add via_arc_point and via_helix_point to clause 5 to support high-speed milling operations.' In annexK, please verify 'Add additive_type_operation to clause 5 to support mapping of Am_operation and subtypes.' In annexK, please verify 'Change the geometric dimensioning and tolerancing UoF to reference the following feature application object definitions and mappings from ISO 10303-1050 and ISO 10303-1051 rather than define them in clause 4 and 5.' In annexK, please verify 'Remove definitions in clause 5 for directed_dimensional_location, externally_defined_dimension_definition, and placed_datum_target_feature.' In annexK, please verify 'The information requirements described in clause 4 have not changed.' In annexK, please verify 'The application interpreted model described in clause 5 has not changed.' In clause5, please verify 'This clause specifies how each UoF and application object in clause 4 shall be represented by one or more AIM EXPRESS constructs given by Annex A.' In introduction, please verify 'The information requirements of the application are specified in clause 4 using terminology appropriate to the application.' In annexK, please verify 'The Process_property name has been reused for an application object defined by the Executable UoF that extends an Executable with additional information.' In clause4, please verify 'The executable UoF specifies the information necessary to describe control flow of a machining program as well as the non-machining actions that may be performed by a numerical control.' In clause4, please verify 'The following application objects are used by the executable UoF, and shall be as defined by ISO 14649-10.' In clause4, please verify 'In addition, the following application objects are used by the executable UoF, and shall be as defined by this document' In clause4, please verify 'The following application objects are used by the executable UoF, and shall be as defined by ISO 10303-1526.' In clause6, please verify 'This conformance class includes the following application objects and their supertypes from the executable UoF' In clause6, please verify 'This conformance class includes everything specified by CC1, plus all remaining application objects from the toolpath UoF and the Parallel application object from the executable UOF.' In clause6, please verify 'This conformance class includes everything specified by CC2, plus all application objects from the manufacturing feature and manufacturing feature for turning UOFs and all remaining application objects from the executable UoF.' |  | RESOLVED (ED) - Capitalized usage |
| SC4QC-7 |  | 4, 5, D, J,K |  | Ed | #7 - Checking spelling typo. annexD; preceeding ==> preceding, proceeding. annexJ; implementors ==> implementers. annexJ; implementors ==> implementers. annexJ; REAM ==> STREAM. annexJ; REAM ==> STREAM. annexK; gage ==> gauge. clause4; contstraints ==> constraints. clause4; gage ==> gauge. clause4; fullfill ==> fulfill, fulfil. clause5; exemple ==> example. clause5; meaurement ==> measurement. clause5; thse ==> these, those, this. clause5; sufficent ==> sufficient. clause5; sucessfully ==> successfully. clause5; definied ==> defined. clause5; propery ==> property, properly. clause5; propery ==> property, properly. clause5; perfoms ==> performs. clause5; preceeding ==> preceding, proceeding. clause5; preceeding ==> preceding, proceeding. clause6; preceeding ==> preceding, proceeding. |  | RESOLVED (ED) - REAM and gage are intended terms, corrected other typos |
| SC4QC-8 |  |  |  |  | Checking if a reference is given both dated and undated in the document. Please verify if the reference ISO 10303-21:2016, ISO 10303-21 should be added dated or undated in the reference list Please verify if the reference ISO 14649-11:2004, ISO 14649-11 should be added dated or undated in the reference list Please verify if the reference ISO 14649-12:2005, ISO 14649-12 should be added dated or undated in the reference list Please verify if the reference ISO 14649-1:2003, ISO 14649-1 should be added dated or undated in the reference list |  | RESOLVED (ED) - Dated references are all in notes or examples. The undated references remain in the reference list. |